# VoE — Video over Ethernet IP

# Product Requirements Document

| Version | 0.2 (Draft) |
| --- | --- |
| Supersedes | 1.0 Draft — renumbered; a draft is not a 1.0 |
| Target board | Numato Mimas A7 V3.1 |
| Target part | xc7a50tfgg484-1 |
| Owner | Development owner: Claude (IP development) |
| Status | Hardware baseline verified against schematic; awaiting sign-off on open decisions |

## Revision history

| Ver | Date | Change |
| --- | --- | --- |
| 1.0 Draft | — | Initial skeleton |
| 0.2 Draft | 2026-09-11 | Renumbered. Added quantified targets, wire format, clocking and CDC plan, verification strategy, resource budget, risk register, phase exit criteria, requirement IDs. Hardware section rebuilt on verified schematic and board-file evidence. |

## Glossary

| Term | Meaning |
| --- | --- |
| AXIS | AXI4-Stream |
| CDC | Clock domain crossing |
| IFG | Inter-frame gap |
| MTU | Maximum transmission unit |
| RGMII | Reduced gigabit media-independent interface |
| SOF / EOF | Start / end of frame |
| TPG | Test pattern generator |

# 1. Executive summary

VoE is a standalone FPGA IP block that streams generated video over IPv4/UDP

from a Mimas A7 V3.1 to a host PC. The emphasis is modular RTL, verification

rigour, and hardware validation over the board's onboard gigabit PHY.

This revision exists because v1.0 Draft specified no numbers. It named the right

modules but contained nothing a test could pass or fail against. Every

requirement below is either measurable or explicitly marked as an open decision.

# 2. Goals and non-goals

## 2.1 Goals

| ID | Goal |
| --- | --- |
| G-1 | A modular, reusable video streaming pipeline in synthesisable RTL |
| G-2 | IPv4/UDP over Ethernet, generated by our own RTL, not vendor IP |
| G-3 | Every module verified with a self-checking testbench and a coverage model |
| G-4 | Demonstrated on hardware against a standard host NIC and Wireshark |

## 2.2 Non-goals (Phase 1)

HDMI input or output, camera capture, H.264 or any compression, Linux or any

software stack on the FPGA, PCIe, TCP, multicast, IGMP, jumbo frames, DDR3

frame buffering, and 10/100 Mbit fallback operation.

# 3. Hardware baseline (verified)

Full detail in hw/mimas_a7_v3_reference.md. The facts that shape requirements:

| Item | Value | Source |
| --- | --- | --- |
| FPGA | xc7a50tfgg484-1 | Numato board files |
| BRAM | ~2.7 Mb (~337 KB) | Part data |
| PHY | RTL8211E-VB-CG, RGMII | Schematic sheet 11 |
| PHY address | Probably 5 — confirm by MDIO scan | Strap resistors |
| RGMII delay straps | TXDLY and RXDLY both unpopulated | Schematic sheet 11 |
| PHY CLK125 output | Not connected | Schematic sheet 11 |
| Reference clock | 100 MHz oscillator, pin H4 | Board files |

Three consequences drive the design and are not negotiable:

H-1 — No on-chip frame buffer is possible. A 720p RGB888 frame is 2.76 MB

against ~337 KB of BRAM. Even 640×480 RGB888 is 921 KB. VoE Phase 1 is

line-buffered; the phrase "frame buffer" is removed from the architecture.

H-2 — The FPGA supplies the entire RGMII skew budget. Both PHY delay straps

have their pull-up and pull-down footprints marked DNP, so internal delay is not

merely off, it is undetermined by the board. Nothing may assume a 2 ns PHY

delay.

H-3 — TX skew must come from an MMCM phase shift. Artix-7 has HR banks only

and therefore no ODELAYE2. RX skew uses IDELAYE2 + IDELAYCTRL at 200 MHz,

which HR banks do support.

# 4. Video format and bandwidth budget

## 4.1 Targets

|  | Phase 1 target | Stretch |
| --- | --- | --- |
| Resolution | 640×480 | 1280×720 |
| Frame rate | 60 fps | 30 fps |
| Pixel format | RGB888 | RGB888 |
| Payload rate | 442.4 Mbps | 663.6 Mbps |
| On-wire rate | 480.2 Mbps (48% of 1 GbE) | 720.2 Mbps (72%) |
| Packets/frame | 960 | 2,880 |
| Packet rate | 57,600 pps | 86,400 pps |

1080p30 4:2:2 needs 995 Mbps of payload before overhead and does not fit a

gigabit link. It is out of scope and will stay out of scope without compression.

## 4.2 Packet arithmetic

Payload is fixed at 320 pixels = 960 bytes, chosen so that both target

widths divide evenly: 640/320 = 2 packets per line, 1280/320 = 4. Lines never

straddle packets, which keeps the receiver's reassembly trivial and the RTL's

address generation free of division.

| Layer | Bytes |
| --- | --- |
| Pixel payload | 960 |
| VoE application header | 16 |
| UDP header | 8 |
| IPv4 header | 20 |
| Ethernet header | 14 |
| FCS | 4 |
| Frame on wire | 1022 |
| Preamble + SFD + IFG | 20 |
| Slot | 1042 |

Overhead is 82/1042 = 7.9%, and the 1022-byte frame sits comfortably inside

a 1500-byte MTU with no fragmentation and no jumbo-frame configuration on the

host.

# 5. Wire format

## 5.1 VoE application header (16 bytes, network byte order)

| Offset | Size | Field | Notes |
| --- | --- | --- | --- |
| 0 | 4 | frame_id | Increments per frame, wraps |
| 4 | 2 | line_num | 0-based |
| 6 | 2 | pixel_off | First pixel index within the line |
| 8 | 2 | pixel_cnt | Pixels in this packet |
| 10 | 2 | width | Frame width |
| 12 | 2 | height | Frame height |
| 14 | 1 | fmt | 0x01 = RGB888 |
| 15 | 1 | flags | bit0 SOF, bit1 EOL, bit2 EOF |

width, height and fmt are repeated in every packet rather than sent once

per frame. It costs 5 bytes out of 1042 and makes the receiver stateless —

it can join a stream mid-frame, and it needs no out-of-band configuration to

decode. That trade is deliberate.

## 5.2 Protocol requirements

| ID | Requirement |
| --- | --- |
| REQ-WF-1 | IPv4 header checksum shall be computed and correct |
| REQ-WF-2 | UDP checksum shall be transmitted as zero (permitted over IPv4) |
| REQ-WF-3 | Frames shorter than 64 bytes shall be padded to 64 |
| REQ-WF-4 | IFG shall be at least 96 bit times |
| REQ-WF-5 | FCS shall be appended and correct |
| REQ-WF-6 | No IP fragmentation shall be emitted |

REQ-WF-3 is called out because it is the classic omission: a short final packet

is a runt on the wire and real NICs drop it silently.

# 6. Architecture

TPG -> AXIS (24-bit) -> Line Buffer -> Packetizer -> UDP/IP -> MAC TX -> RGMII -> PHY -> RJ45

^

ARP + ICMP responder <- MAC RX <---------+

No frame buffer, per H-1. The RX path in Phase 1 is control-plane only.

## 6.1 Clock domains

| Domain | Frequency | Source | Purpose |
| --- | --- | --- | --- |
| clk_sys | 100 MHz | Oscillator H4 | Control, MDIO, status |
| clk_tx | 125 MHz | MMCM | RGMII TX, datapath |
| clk_tx90 | 125 MHz, 90° | MMCM | TXC ODDR — the TX skew source (H-3) |
| clk_ref | 200 MHz | MMCM | IDELAYCTRL reference |
| clk_rx | 125 MHz | Recovered from PHY | RGMII RX — independent domain |

MMCM recipe, copied from Numato's working design: VCO 1000 MHz (M=10, D=1),

divides of 8 and 5 giving 125 and 200 MHz exactly.

## 6.2 CDC plan

| ID | Requirement |
| --- | --- |
| REQ-CDC-1 | All data crossings shall use async FIFOs with gray-coded pointers |
| REQ-CDC-2 | All control/status crossings shall use two-flop synchronisers |
| REQ-CDC-3 | Every crossing shall be listed in a CDC table in the design docs |
| REQ-CDC-4 | Crossings shall carry set_max_delay -datapath_only constraints |
| REQ-CDC-5 | report_cdc shall be clean of critical findings, and run in CI |

Phase 1 simplification with a deliberate hook: the TPG runs in clk_tx and

is throttled to the target frame rate, which removes the pixel-clock crossing

entirely. The async FIFO at that boundary is still instantiated, same-clock, so

that adding a real pixel clock in Phase 2 — for camera or HDMI input — is a

configuration change and not a rework.

## 6.3 Datapath width

GMII-equivalent throughput at 8 bits × 125 MHz is exactly 1 Gbps, i.e. zero

margin. The internal stream therefore runs **32-bit at 125 MHz (4 Gbps, 4×

headroom)** downstream of the packetizer, with tkeep for tail bytes. The TPG

output stream is 24-bit, one pixel per beat, converted by an explicit width

adapter. Making the conversion a named module rather than an implicit packing

rule keeps the awkward 3-bytes-into-4 arithmetic in one verifiable place.

# 7. Functional requirements

## 7.1 Test pattern generator

| ID | Requirement |
| --- | --- |
| REQ-TPG-1 | Generate colour bars, a moving gradient, and a per-frame counter pattern |
| REQ-TPG-2 | Resolution and frame rate shall be parameters, not hard-coded constants |
| REQ-TPG-3 | Output shall be AXI4-Stream, 24-bit, with tuser marking SOF and tlast marking end of line |
| REQ-TPG-4 | Patterns shall be reproducible by the Python golden model, bit-exact |

## 7.2 Line buffer and packetizer

| ID | Requirement |
| --- | --- |
| REQ-PKT-1 | Buffer at least 2 lines, sized from the width parameter |
| REQ-PKT-2 | Emit fixed 320-pixel payloads; no payload shall straddle a line |
| REQ-PKT-3 | Populate the 16-byte header per §5.1 |
| REQ-PKT-4 | Assert EOF on the final packet of each frame |
| REQ-PKT-5 | Apply backpressure upstream rather than dropping on downstream stall |

## 7.3 UDP/IP

| ID | Requirement |
| --- | --- |
| REQ-IP-1 | Emit valid IPv4 + UDP per §5.2 |
| REQ-IP-2 | Source/destination MAC, IP and port shall be runtime-writable registers |
| REQ-IP-3 | Respond to ARP requests for the configured IP |
| REQ-IP-4 | Issue ARP requests to resolve the destination MAC |
| REQ-IP-5 | Respond to ICMP echo requests |

REQ-IP-5 is not decoration. Being able to ping the board before any video flows

separates "the link works" from "my packetizer is wrong", and that split is

worth a great deal during bring-up.

## 7.4 Ethernet MAC and RGMII

| ID | Requirement |
| --- | --- |
| REQ-MAC-1 | Full-duplex 1000BASE-T only; no 10/100 fallback in Phase 1 |
| REQ-MAC-2 | TXC shall be driven by an ODDR clocked from clk_tx90 (H-3) |
| REQ-MAC-3 | RX shall use IDELAYE2 on rxd[3:0] and rx_ctl, with IDELAYCTRL at 200 MHz |
| REQ-MAC-4 | IDELAY tap value shall be runtime-adjustable for eye characterisation |
| REQ-MAC-5 | Provide an MDIO master; scan addresses 0–31 and report the PHY found |
| REQ-MAC-6 | Read back and report the PHY's latched strap configuration |
| REQ-MAC-7 | Hold PHY reset for the specified minimum, then wait ≥10 ms before the first MDIO transaction |
| REQ-MAC-8 | Report link up/down, speed and duplex as status registers |

REQ-MAC-4 exists because H-2 leaves us no inherited delay value and no strap to

start from. The tap centre must be found empirically, so it must be adjustable

without a rebuild.

## 7.5 Statistics

| ID | Requirement |
| --- | --- |
| REQ-STAT-1 | Count TX frames, TX bytes, and frames generated |
| REQ-STAT-2 | Count RX frames, CRC errors, and ARP/ICMP handled |
| REQ-STAT-3 | Count FIFO overflow and underflow events |
| REQ-STAT-4 | Counters shall be readable without disturbing the stream |

## 7.6 Host receiver

| ID | Requirement |
| --- | --- |
| REQ-RX-1 | Python receiver shall reassemble frames from the §5.1 header |
| REQ-RX-2 | Detect and report lost, duplicated and reordered packets |
| REQ-RX-3 | Verify received frames against the golden model bit-exactly |
| REQ-RX-4 | Report achieved throughput, frame rate and loss rate |

# 8. Performance requirements

| ID | Requirement | Target |
| --- | --- | --- |
| REQ-PERF-1 | Sustained on-wire throughput at 640×480@60 | ≥480 Mbps |
| REQ-PERF-2 | Dropped frames over a 10-minute run | 0 |
| REQ-PERF-3 | Packet loss, direct point-to-point link, 10 minutes | ≤1×10⁻⁶ |
| REQ-PERF-4 | Latency, first pixel in to first byte on wire | ≤50 µs |
| REQ-PERF-5 | Frame integrity, checksum vs golden model | 100% of frames |
| REQ-PERF-6 | Link recovery after cable pull | ≤5 s, no manual reset |

REQ-PERF-4 is achievable by a wide margin precisely because there is no frame

buffer: filling one 320-pixel payload takes ~17 µs at the Phase 1 pixel rate and

serialising it takes ~8 µs.

# 9. Resource and timing budget

| Resource | Budget | Available (XC7A50T) | Headroom |
| --- | --- | --- | --- |
| LUT | ≤12,000 | 32,600 | 63% |
| FF | ≤12,000 | 65,200 | 82% |
| BRAM36 | ≤20 | 75 | 73% |
| DSP48 | 0 | 120 | — |
| MMCM | 1 | 6 | — |

| ID | Requirement |
| --- | --- |
| REQ-TIM-1 | All clocks shall meet timing with WNS ≥ 0.5 ns |
| REQ-TIM-2 | No timing exception shall be added without a comment justifying it |
| REQ-TIM-3 | Asynchronous clock groups shall be declared, not left inferred |

# 10. Verification

## 10.1 Strategy

Primary: cocotb + Verilator. Free, fast, runs in CI on every push, and lets

the Python golden model be shared verbatim between simulation and the host

receiver — one model, so a disagreement between sim and hardware is a real bug

rather than two models drifting.

Secondary: Vivado XSim for anything touching Xilinx primitives — ODDR,

IDELAYE2, IDELAYCTRL, MMCM — and for post-synthesis and post-implementation

runs. Verilator cannot model these.

The cost is honest: Verilator's SystemVerilog assertion support is partial, so

protocol checks are written as cocotb monitors rather than concurrent SVA. If a

Questa or VCS licence is available, the SVA-based plan in v1.0 Draft becomes

viable and this decision should be revisited. **This is an open decision — see

§14.**

## 10.2 Requirements

| ID | Requirement |
| --- | --- |
| REQ-VER-1 | Every RTL module shall have a self-checking testbench |
| REQ-VER-2 | A written coverage plan shall define bins before coverage is measured |
| REQ-VER-3 | Functional coverage ≥95% against that plan |
| REQ-VER-4 | Code coverage — line, branch, toggle, FSM — reported separately |
| REQ-VER-5 | Cross-coverage of resolution × packet position × error type |
| REQ-VER-6 | A scoreboard shall compare against the Python golden model |
| REQ-VER-7 | Error injection: packet loss, CRC error, FIFO overflow, FIFO underflow, mid-frame reset, sustained backpressure |
| REQ-VER-8 | Constrained-random regression shall run in CI on every push |
| REQ-VER-9 | Verilator lint shall be clean |
| REQ-VER-10 | Each requirement ID shall map to at least one test |

REQ-VER-2 is the substantive change from v1.0 Draft. "95% functional coverage"

without a defined coverage model measures nothing — it is a number that can be

reached by writing bins around whatever the tests already do.

# 11. Hardware validation

| ID | Requirement |
| --- | --- |
| REQ-HV-1 | PHY detected by MDIO scan, address reported |
| REQ-HV-2 | Link up at 1000 Mbit full duplex, reported in status |
| REQ-HV-3 | Board answers ping |
| REQ-HV-4 | ARP resolves correctly from the host |
| REQ-HV-5 | Wireshark shows well-formed IPv4/UDP with correct checksums |
| REQ-HV-6 | Host receiver reconstructs frames matching the golden model |
| REQ-HV-7 | RX eye characterised by IDELAY tap sweep; margin recorded |
| REQ-HV-8 | Performance targets of §8 measured and recorded |

REQ-HV-7 produces the single most valuable artifact of bring-up: a tap-vs-error

curve showing where the sampling window actually sits and how much margin we

have. Given H-2, this is measured, not inherited.

# 12. Risk register

| ID | Risk | L | I | Mitigation |
| --- | --- | --- | --- | --- |
| R-1 | RGMII skew. No PHY delay strapped (H-2), no value to inherit | H | H | Runtime-adjustable IDELAY (REQ-MAC-4); ILA eye scan; Numato reference bitstream as known-good comparison |
| R-2 | RTL8211E is poorly documented. Realtek does not publish the datasheet | H | M | Use drivers/net/phy/realtek.c as the reference for extended-page registers; read straps back over MDIO (REQ-MAC-6) |
| R-3 | C143 (27 pF on RX_CLK) degrades the RX clock edge | M | M | Verify populated on the physical board; fold into the tap sweep of REQ-HV-7 |
| R-4 | PHY address assumption (5) is wrong | L | L | Scan all 32 addresses (REQ-MAC-5) |
| R-5 | Verilator's partial SVA support weakens protocol checking | M | M | cocotb monitors instead of SVA; revisit if a Questa licence appears |
| R-6 | Host NIC coalescing or offload distorts measurements | M | M | Disable offload on the test NIC; direct point-to-point link only |
| R-7 | Timing closure at 125 MHz on a -1 speed grade part | L | M | 4× datapath headroom (§6.3); early trial implementation in Phase 3 |
| R-8 | Scope creep toward DDR3, HDMI, compression | M | H | §2.2 is binding; changes require a PRD revision |

# 13. Development phases

Each phase has an exit criterion. A phase is not complete because its work is

done; it is complete because its criterion is demonstrated.

| # | Phase | Exit criterion |
| --- | --- | --- |
| 0 | De-risk | Numato reference bitstream flashed; telnet echo answers at 192.168.1.10:7. Board, PHY, cable and host NIC all proven before any VoE RTL is debugged. |
| 1 | Infrastructure | Repo, CI, lint, cocotb harness, golden model skeleton. CI green on an empty design. |
| 2 | Video generator | TPG matches golden model bit-exactly in simulation. |
| 3 | RGMII + MDIO | PHY detected, link up at 1 Gbit, straps read back. Trial implementation meets timing. |
| 4 | MAC + ARP/ICMP | Board answers ping. Wireshark shows well-formed frames. |
| 5 | UDP/IP + packetizer | Valid video packets on the wire, correct checksums, no runts. |
| 6 | End-to-end | Host receiver reconstructs frames matching the golden model. |
| 7 | Verification | Coverage targets met; full regression green; requirement-to-test matrix complete. |
| 8 | Hardware validation | All §8 performance targets measured and met; eye margin recorded. |
| 9 | Documentation + release | Tagged release, integration guide, licence declared. |

Phase 0 is new and is placed first deliberately. It costs an afternoon and it

means that when our own RGMII does not link up, we already know the hardware is

not the reason.

# 14. Open decisions

| ID | Decision | Recommendation |
| --- | --- | --- |
| D-1 | Simulator: cocotb+Verilator vs Questa/VCS | cocotb + Verilator, unless a licence is already available |
| D-2 | Own MAC vs verilog-ethernet (MIT) | Own MAC. This is an IP-ownership project; a third-party MAC would hollow out G-2. Keep verilog-ethernet as a reference to compare against when debugging. |
| D-3 | Licence for the released IP | Undecided — needs an answer before Phase 9 |
| D-4 | Does Phase 2 target camera input or HDMI input first? | Defer; §6.2 keeps the hook open either way |

D-1 and D-2 are recorded as owner recommendations, not settled facts. Both are

reversible until Phase 3; after that, switching either is expensive.

# 15. Deliverables

RTL source · cocotb testbenches and golden model · coverage and lint reports ·

CDC table · Vivado project and constraints · bitstream · Python host receiver ·

requirement-to-test traceability matrix · bring-up guide including the IDELAY

tap-sweep results · integration guide · tagged release.